GENERAL WASTE SEGREGATION GUIDE

Purpose: This guide provides clear instructions on how to properly identify and dispose of general waste — materials that cannot be recycled, composted, or classified as hazardous. While general waste is destined for landfill, proper segregation ensures that recyclable, biodegradable, and hazardous materials are not mixed in, reducing overall waste volume and environmental impact.

⚠️ IMPORTANT NOTICE

General waste should always be your last resort. Before placing any item in the general waste bin, ask yourself: Can this be recycled? Can this be composted? Does this require special hazardous disposal? Only if the answer to all three questions is no should the item go into the general waste bin. Reducing what goes into general waste is one of the most effective ways to help the environment.

1. CONTAMINATED FOOD WRAPPERS AND PACKAGING

Color Code: Black or gray bin

Accepted items: Greasy pizza boxes that cannot be separated into clean and soiled portions, food-soiled aluminum foil, plastic food wrapping with heavy food residue that cannot be rinsed clean, used paper napkins and tissues soiled with food, greasy paper bags, chip and snack bags, candy wrappers, wax paper used for food wrapping, and multilayer food packaging such as juice pouches and snack sachets.

Why these cannot be recycled: Food contamination is the primary reason these items cannot enter the recycling stream. Grease and food residue break down paper fibers and contaminate entire batches of recyclable material at processing facilities. Multilayer packaging — such as juice pouches and snack wrappers — is made from several bonded materials including plastic, aluminum, and paper that cannot be separated and therefore cannot be recycled through standard processes.

Not to be confused with: Clean aluminum foil, which can be recycled. Clean plastic containers, which can be rinsed and recycled. Cardboard boxes with only minor grease stains on a small area — the clean portion can be torn off and recycled separately.

How to prepare: Wrap soiled food packaging tightly to contain odors and prevent attracting pests. Do not leave food residue exposed inside the bin. If possible, scrape off excess food from wrappers and packaging before disposing — this reduces odor and keeps your bin cleaner. Tie general waste bags securely before placing them in the bin.

Why it matters: Contaminated packaging placed in recycling bins is one of the leading causes of recycling contamination globally. Even a small amount of grease or food residue can render an entire batch of otherwise recyclable material unusable, sending it to landfill anyway. Placing contaminated items directly into general waste — rather than wishful recycling — is always the more responsible choice.

2. NON-RECYCLABLE PLASTICS

Color Code: Black or gray bin

Accepted items: Plastic bags and cling film, Styrofoam and foam packaging, plastic cutlery and single-use straws, plastic straws, coffee cup lids, plastic drinking cups, broken plastic toys and household items, plastic-coated wire and cables, multilayer plastic packaging, plastic bubble wrap, and plastic bottle caps that are too small to be processed.

Why these cannot be recycled: These plastics are either made from resin codes not accepted by standard recycling facilities (such as #6 polystyrene and #7 mixed plastics), are too small or lightweight to be sorted by machinery, or are made from multiple bonded materials that cannot be separated. Plastic bags in particular cause significant problems at recycling facilities — they wrap around sorting machinery, causing equipment breakdowns and facility shutdowns.

Not to be confused with: Plastic bottles and rigid plastic containers coded #1 and #2, which are widely accepted in recycling bins. Plastic bags can sometimes be returned to supermarket collection points for specialist recycling rather than placed in general waste.

How to prepare: Collect small non-recyclable plastic items together in one bag before placing them in the general waste bin. This prevents small plastics from scattering and potentially entering waterways as litter. Never burn plastic waste — burning plastic releases toxic dioxins and furans into the air, which are linked to cancer and severe respiratory illness.

Why it matters: Non-recyclable plastics represent a significant waste management challenge. While they must currently go to landfill, reducing their use in the first place — by choosing reusable bags, refusing single-use cutlery, and selecting products with recyclable packaging — is the most effective long-term solution.

3. NON-RECYCLABLE ORGANICS

Color Code: Black or gray bin

Accepted items: Meat and fish bones in large quantities (if not accepted by your local composting facility), fats, oils, and grease in large amounts, pet waste and cat litter, disposable diapers and sanitary products, heavily soiled or contaminated organic material, animal carcasses (small household pets — check local regulations), and treated or preserved organic materials such as leather.

Why these cannot be composted: Large meat bones take an extremely long time to break down and can attract vermin to compost piles. Pet waste contains pathogens and parasites — such as toxoplasma, salmonella, and E. coli — that can survive the composting process and contaminate the resulting compost, making it unsafe for use on food-growing soil. Disposable diapers and sanitary products, while partially organic in their absorbent materials, are bonded with plastics and superabsorbent polymers that do not biodegrade and contaminate compost batches. Fats and cooking grease in large amounts create anaerobic conditions in compost that produce foul odors and attract pests.

Not to be confused with: Small amounts of cooked food, including small fish bones, which some composting facilities do accept. Always check with your local composting or green waste program for their specific guidelines.

How to prepare: Double-bag pet waste and soiled diapers before placing them in the general waste bin to contain pathogens, odors, and leaks. Allow cooking grease and fat to cool and solidify before sealing it in a container and placing it in general waste — never pour liquid grease or fat into any bin, as it solidifies and causes blockages. Keep these items separate from dry general waste to manage odors effectively.

Why it matters: Non-recyclable organics are a significant source of odor, pest attraction, and methane production in landfills. While they cannot be composted safely at home, some municipalities operate high-temperature industrial composting or anaerobic digestion facilities that can process these materials safely. Check with your local waste authority to find out if such services are available in your area.

4. PERSONAL HYGIENE AND SANITARY PRODUCTS

Color Code: Black or gray bin

Accepted items: Disposable diapers and nappies, sanitary pads and tampons, panty liners, incontinence pads, cotton swabs and cotton balls (used), bandages and medical dressings (non-sharps), face masks (non-industrial), disposable gloves (non-hazardous), wet wipes and baby wipes, dental floss, and razors (without detachable blades).

Why these cannot be recycled or composted: These products are designed for single use and are made from a combination of materials including plastic, superabsorbent polymers, adhesives, and organic fibers that cannot be separated or processed through standard recycling or composting systems. They also carry biological contamination — bodily fluids, bacteria, and pathogens — that makes them unsuitable for any recovery process.

Not to be confused with: Used medical sharps such as needles and lancets, which are hazardous waste and must be placed in an approved sharps container — never in the general waste bin. Industrial or N95 respirator masks used in hazardous environments, which may also require special disposal depending on what they have been exposed to.

How to prepare: Always wrap used sanitary products and diapers in their own packaging or a small plastic bag before placing them in the bin. This contains odors, prevents the spread of bacteria, and maintains hygiene for waste handlers. Never flush sanitary products, wet wipes, or cotton items down the toilet — they do not dissolve in water and are the leading cause of sewer blockages and fatbergs in municipal water systems.

Why it matters: Wet wipes and sanitary products flushed down toilets cause enormous damage to sewage infrastructure and marine environments. A single wet wipe can take 100 years to break down in the ocean. Disposing of these items correctly in general waste — while imperfect — is far less harmful than flushing them.

5. BROKEN AND NON-REPAIRABLE HOUSEHOLD ITEMS

Color Code: Black or gray bin (bulky items may require separate collection)

Accepted items: Broken ceramic dishes and pottery, broken mirrors and window glass (wrapped safely), torn or worn-out synthetic clothing and footwear, broken umbrellas, worn-out synthetic rugs and carpets, broken toys made of mixed materials, non-functioning small appliances with no e-waste components, worn-out mattresses and pillows (check for bulky waste collection), and broken or expired plastic storage containers.

Why these cannot be recycled: Most of these items are made from mixed or composite materials that cannot be separated into single-material streams for recycling. Broken ceramics and pottery have a different chemical composition from glass and contaminate glass recycling batches. Synthetic textiles such as polyester and nylon do not biodegrade and are generally not accepted in standard textile recycling programs, though specialist recyclers for synthetic fabrics do exist in some areas.

How to prepare: Wrap broken glass, mirrors, and sharp ceramic pieces in several layers of newspaper or thick cloth, seal them in a box or sturdy bag, and clearly label them "Sharp — Handle with Care" to protect waste handlers from injury. For bulky items such as mattresses, furniture, and large appliances, contact your local waste management authority for scheduled bulky waste collection services rather than placing them on the street or in regular bins.

Why it matters: Improperly discarded sharp items such as broken glass and ceramics are among the most common causes of injury to waste collection and sorting workers. Taking the extra step to wrap and label sharp waste protects the people who handle your waste every day.

6. MISCELLANEOUS NON-RECYCLABLE MATERIALS

Color Code: Black or gray bin

Accepted items: Rubber bands and elastic, sticky tape and masking tape, used stickers and labels, correction fluid (dried), carbon paper, laminated documents, photographs, receipts and thermal paper, chewing gum, candle wax and melted wax, broken crayons (wax-based), polystyrene packing peanuts, and spent matches.

Why these cannot be recycled: These items are made from materials that are either too small to be captured by sorting equipment, are made from chemically treated or composite substances that interfere with recycling processes, or exist in quantities too small to justify the energy cost of processing. Thermal paper such as receipts contains BPA or BPS — chemical coatings that contaminate paper recycling batches and pose health risks.

How to prepare: Collect small miscellaneous non-recyclable items together in a small bag or container before placing them in the general waste bin. This prevents small items from scattering in the environment as litter. Dispose of spent candles and wax by allowing them to cool and harden completely before wrapping and binning.

Why it matters: While these items individually seem insignificant, they collectively represent a large volume of waste. Being mindful of purchasing products with less packaging and fewer non-recyclable components is the most effective way to reduce this category of waste over time.

GENERAL WASTE REDUCTION TIPS

The best general waste is waste that was never created. Here are practical ways to reduce how much general waste your household or workplace generates:

Refuse: Say no to single-use items, unnecessary packaging, promotional freebies, and disposable products whenever possible.

Reduce: Buy only what you need. Purchase products in larger quantities to reduce packaging waste per unit. Choose products with minimal or recyclable packaging.

Reuse: Opt for reusable alternatives — cloth bags instead of plastic bags, reusable water bottles instead of single-use ones, rechargeable batteries instead of disposable ones, and washable cloths instead of paper towels.

Repair: Before discarding broken items, consider whether they can be repaired. Many communities have repair cafés, cobblers, tailors, and electronics repair shops that can extend the life of household items significantly.

Rethink: Before purchasing a new product, consider its entire lifecycle — how it will be used, how long it will last, and how it will be disposed of. Choosing durable, long-lasting products over cheap disposable alternatives reduces long-term waste significantly.

QUICK REFERENCE TABLE

| Item | General Waste | Recycle | Compost | Hazardous |
| --- | --- | --- | --- | --- |
| Greasy food wrappers | Yes | No | No | No |
| Plastic bags and cling film | Yes | No* | No | No |
| Styrofoam packaging | Yes | No | No | No |
| Pet waste and cat litter | Yes | No | No | No |
| Disposable diapers | Yes | No | No | No |
| Sanitary products | Yes | No | No | No |
| Wet wipes and baby wipes | Yes | No | No | No |
| Broken ceramics and pottery | Yes | No | No | No |
| Synthetic clothing and fabric | Yes | No | No | No |
| Thermal paper receipts | Yes | No | No | No |
| Photographs | Yes | No | No | No |
| Laminated documents | Yes | No | No | No |
| Rubber bands and tape | Yes | No | No | No |
| Chewing gum | Yes | No | No | No |
| Large meat and fish bones | Yes | No | No** | No |
| Cooking grease (solidified) | Yes | No | No | No |

*Plastic bags may be returned to supermarket drop-off points for specialist recycling in some areas. **Some industrial composting facilities accept meat and bones — check local guidelines.

GENERAL REMINDERS

- General waste is the last resort. Always exhaust recycling, composting, and donation options before placing items in the general waste bin.

- Never contaminate other bins. Placing general waste into recycling or compost bins ruins entire batches of otherwise recoverable material.

- Bag your waste properly. Always use a tied, sealed bag inside your general waste bin to contain odors, prevent pests, and maintain hygiene for waste collection workers.

- Label sharp waste. Always wrap and clearly label broken glass, ceramics, and sharp objects before placing them in the bin to protect waste handlers.

- Never burn general waste. Open burning of waste releases toxic pollutants into the air and is illegal in many municipalities.

- Reduce first. The most powerful action you can take is to reduce the amount of general waste you produce in the first place, by refusing single-use items and choosing reusable and recyclable alternatives.

- Check local guidelines. Accepted general waste items and collection schedules vary by city and municipality. Contact your local solid waste management office for specific local rules.