RECYCLABLE MATERIALS SEGREGATION GUIDE

Purpose: This guide provides clear instructions on how to properly sort and prepare recyclable materials before disposal. Following these guidelines helps reduce landfill waste, conserve natural resources, and support a cleaner environment.

1. PAPER

Color Code: Blue bin

Accepted items: Newspapers, magazines, office paper, printer paper, envelopes (without plastic windows), paper bags, junk mail, and shredded paper (placed in a sealed paper bag).

Not accepted: Greasy or food-soiled paper, paper towels, tissues, napkins, wax-coated paper, glossy wrapping paper, and wet or water-damaged paper.

How to prepare: Keep paper clean and dry at all times. Remove plastic windows from envelopes. Flatten and stack papers neatly. Place shredded paper in a sealed paper bag to prevent it from scattering.

Why it matters: Recycling one ton of paper saves approximately 17 trees and 7,000 gallons of water. Recycled paper can be processed and reused up to 5–7 times before the fibers become too short for further use.

2. CARDBOARD

Color Code: Brown bin

Accepted items: Shipping boxes, cereal boxes, shoe boxes, toilet paper roll tubes, egg cartons, clean food boxes, tissue boxes, and moving boxes.

Not accepted: Pizza boxes with grease stains, wax-coated or laminated cardboard, wet or moldy cardboard, juice boxes, and milk cartons (which have a plastic or foil lining).

How to prepare: Always flatten boxes completely before placing them in the bin. Remove all tape, staples, and packing materials. If a pizza box has a greasy bottom, cut or tear off the clean top half and recycle that portion only. Store cardboard in a dry area to prevent moisture damage.

Why it matters: Recycling one ton of cardboard saves over 9 cubic yards of landfill space. Cardboard that ends up in landfills decomposes slowly and produces methane, a greenhouse gas more potent than carbon dioxide.

3. GLASS BOTTLES AND JARS

Color Code: Green bin

Accepted items: Clear, green, and brown glass bottles; glass food jars; condiment jars; juice bottles; wine and beer bottles; and empty glass medicine bottles.

Not accepted: Window glass, mirrors, eyeglasses, ceramics, porcelain, pottery, lightbulbs (fluorescent or LED), and broken glass mixed with other recyclables.

How to prepare: Rinse bottles and jars to remove major food residue. Labels do not need to be removed as they burn off during processing. Separate metal lids and place them in the metal bin. Place plastic lids in the plastic bin. If glass is broken, bag it separately, seal it securely, and label it "Broken Glass" for safety.

Why it matters: Glass is 100% recyclable and can be recycled endlessly without any loss in quality or purity. Producing glass from recycled material uses 40% less energy than producing it from raw materials. A recycled glass bottle can return to a store shelf as a new bottle in as little as 30 days.

4. CLEAN PLASTIC CONTAINERS

Color Code: Yellow bin

Accepted items (most widely accepted): Plastic water bottles (code #1 PET), soda and juice bottles (#1), milk jugs (#2 HDPE), shampoo and conditioner bottles (#2), cleaning product bottles (#2), and yogurt containers (#2).

Check with your local facility: Food tubs (#5 PP), plastic cups and straws (#5), and mixed plastics (#7) may or may not be accepted depending on your local recycling program.

Not accepted: Plastic bags and film, Styrofoam/foam packaging (#6), dirty or food-coated containers, and plastic utensils or single-use cutlery.

How to prepare: Rinse all containers thoroughly, especially jars and bottles that previously held peanut butter, sauces, or oil. Leave caps on bottles. Lightly crush bottles to save space. Always check the resin code (the number inside the triangle symbol) stamped on the bottom of the container before placing it in the bin.

Why it matters: Plastic takes 400 to 1,000 years to decompose in a landfill. Recycling just 10 plastic bottles saves enough energy to power a laptop computer for approximately 25 hours.

5. METAL CANS AND FOIL

Color Code: Gray or silver bin

Accepted items: Aluminum beverage cans, steel food cans, tin cans, metal jar lids, metal bottle caps, clean aluminum foil, biscuit and cookie tins, and empty aerosol cans.

Not accepted: Full or pressurized aerosol cans, paint cans with dried or wet paint, large scrap metal objects, and aluminum foil with heavy food residue.

How to prepare: Rinse all cans and let them dry. Crush aluminum cans to save bin space (check local rules, as some facilities prefer uncrushed cans). Collect small metal caps and lids inside a steel can, then crimp the top shut before placing in the bin. Ball up aluminum foil into a fist-sized lump so it is not lost during sorting.

Why it matters: Aluminum is infinitely recyclable and recycling it uses 95% less energy than producing new aluminum from raw ore. A recycled aluminum can be back on a store shelf as a new can in as little as 60 days.

6. SPECIAL AND HAZARDOUS ITEMS

Important: These items must NEVER be placed in regular recycling or trash bins. They require special drop-off locations or collection events.

| Item | Where to bring it |
| --- | --- |
| Electronics (phones, laptops, TVs) | E-waste collection drives or electronics stores |
| Batteries (all types) | Hardware stores, supermarkets, or battery drop-off boxes |
| Fluorescent and LED bulbs | Hardware stores or hazardous waste facilities |
| Medicines and drugs | Pharmacy take-back programs |
| Motor oil and paint | Hazardous waste collection facilities |
| Used cooking oil | Biodiesel collection points or local recycling centers |
| Ink cartridges | Office supply stores or manufacturer take-back programs |

Why it matters: These materials contain toxic substances such as lead, mercury, lithium, and cadmium. Improper disposal contaminates soil and groundwater, which can affect entire communities and ecosystems for decades.

GENERAL REMINDERS

- When in doubt, leave it out. One contaminated item can ruin an entire batch of recyclables.

- Clean before you bin. A quick rinse is enough. Food residue is the leading cause of recycling contamination.

- Keep it dry. Moisture damages paper and cardboard and can spread to nearby materials.

- Flatten and compress. Flattened boxes and crushed bottles save space and improve collection efficiency.

- Check local guidelines. Accepted materials vary by city and municipality. When unsure, contact your local waste management office.

- Reduce and reuse first. The most effective way to manage waste is to produce less of it in the first place.